# LCD SUPERTWIST 16 x 4 - RETRO-ECLAIRAGE - AVEC LEDS

| Référence: LCD1604ASL |  |
| --- | --- |

## Caractéristiques

- nombre de caractères: 16 x 4

- caractère: matrice 5 x 7 points

- duty-bias: 1/16 - 1/4

- type: positif

## Spécifications

- rétro-éclairage: LED

- type d'afficheur + couleur: STN jaune-vert

- valeurs:

  - alimentation du driver de l'afficheur (Vdd-Vo):

    - à -20°C: -

    - à 0°C: 4.8V typ.

    - à 25°C: 4.4V typ.

    - à 50°C: 4.1V typ.

    - à 70°C: -

  - alimentation de la logique (Vdd-Vss):

    - à 25°C: 5.0V typ.

  - température de travail: 0°C à +50°C

  - température de stockage: -20°C à +60°C

- mécaniques:

  - dimensions totales: 87.0 x 60.0mm

  - dimensions écran: 62.0 x 25.6mm

  - dimensions des points: 0.55 x 0.55mm

  - dimensions des caractères: 2.95 x 4.75mm

  - module: H2/H1

    - sans rétro-éclairage: 5.1/9.7 ± 0.5mm

    - rétro-éclairage EL: 5.1/9.7 ± 0.5mm

    - rétro-éclairage LED: 8.9/13.5 ± 0.5mm

- broches:

  - power supply (GND): 1 (Vss)

  - power supply (+5V): 2 (Vdd)

  - contrast adjust: 3 (Vo)

  - signal for register selection: 4 (RS)

  - data read & write: 5 (R/)

  - enable signal: 6 (E)

  - data bus line: 7 (DB0)

  - data bus line: 8 (DB1)

  - data bus line: 9 (DB2)

  - data bus line: 10 (DB3)

  - data bus line: 11 (DB4)

  - data bus line: 12 (DB5)

  - data bus line: 13 (DB6)

  - data bus line: 14 (DB7)

  - backlight power supply: 15 (A)

  - backlight power supply: 16 (K)