program mechprothese

'VARIABLE DECLARATION

dim Spier1 as word

dim Spier2 as word

dim Spier3 as word

dim Manual as word

dim Manualdisplay as word

dim Fr as word

dim SP1 as string[6]

dim SP2 as string[6]

dim SP3 as string[6]

dim Frequentie as string[6]

dim V as word

dim A as word

dim T as word

dim periode as word

dim Stifness as string[6]

dim Count as word

dim B as boolean

dim DemoKnop as boolean

dim ManKnop as boolean

dim NopKnop as boolean

'SUBPROCEDURES

sub procedure ReadValues

Manual =  (ADC_read(4))/41  * 10 'read the level of the manual switch

Manualdisplay = (Manual)*10/24

WordToStr(Manualdisplay,Stifness)

Spier1 = (ADC_read(0)-80)*5' 0     'read pressure in Spier1 and conversion to: min 0 mbar (steps of 100mbar) max 3000mbar

WordToStr(Spier1,SP1)  ' convert word to string for display on LCD

'WordToStr(Count,Teller) 'Needed

Spier2 = (ADC_read(1)-80)*5'/ 34 *100     'read pressure in Spier2 and conversion to: min 0 mbar (steps of 100mbar) max 3000mbar

WordToStr(Spier2,SP2)  ' convert word to string for display on LCD

Spier3 = (ADC_read(2)-80)*5'/ 34 * 100    'read pressure in Spier3 and conversion to: min 0 mbar (steps of 100mbar) max 3000mbar

WordToStr(Spier3,SP3)  ' convert word to string for display on LCD

Fr = ADC_read(3) '/ 50    'Read frequentie

WordToStr(Fr,Frequentie)

end sub

sub procedure Display           'write to LCD

Lcd8_out(1,8,SP1)         'value of Spier1

Lcd8_out(2,8,SP2)         'value of Spier2

Lcd8_out(3,4,SP3)         'value of Spier3

'Lcd8_out(4,-1,angle)      'value of Hoek

Delay_ms(100)

end sub

sub procedure welcome           'write welcome text to LCD

Lcd8_out(1,1,"*** welcome! ***")

Lcd8_out(2,1,"  BEENPROTHESE  ")

Lcd8_out(3,-3,"Arman & Max 2007")

Lcd8_out(4,-3,"INITALIZING .")

Delay_ms(1000)

Lcd8_out(4,-3,"INITALIZING ..")

Delay_ms(1000)

Lcd8_out(4,-3,"INITALIZING ...")

Delay_ms(1000)

Lcd8_out(4,-3,"INITALIZING ....")

Delay_ms(1000)

end sub

' START OF THE PROGRAM

'_________________________________________________________________

'_________________________________________________________________

main:

ADCON1 = %10000010  ' configure analog inputs and Vref

TRISA  = %00101111  ' designate PORTA as input (Bit 0-3 :Spier1,2,3,Stifness, Bit 5: Manual) !!!en toch ADC_read(4)!!!

TRISB  = %00000000  ' Port B as output  (Display DB0-DB7)

TRISC  = %00011001  ' 7 => 0 (E,RW,RS,switch2,switch1,  Spier2+spier3,Spier1,switch0)

'    			7  6 5    4      3       2          1        0

Lcd8_Init(PORTB,7,6,5,4,3,2,1,0,PORTC,5,6,7)'initialising Display (standard procedure)   (DB7,...,DB0,RS,R/W,E)

Lcd8_Cmd(LCD_CLEAR)     'clear screen

Delay_ms(1000)            '1sec delay

welcome

Delay_ms(1000)            '1sec delay

Lcd8_out(1,1,"Spier1:       mb")                     'standard procedure to write to LCD

Lcd8_out(2,1,"Spier2:       mb")

Lcd8_out(3,-3,"Spier3:       mb")                   'to syncronise positions you have to scoop 5 places on line 3 and 4

'PREDEFINED VARIABLES

'---------------------

Count = 0

V=0

A=0

DemoKnop=0

ManKnop=0

NopKnop=0

'INITIALIZING PWM

'----------------------

PWM1_init(8000) ' For Spier2 en 3 portC.2

PWM1_start

PWM2_init(8000) ' For Spier1 portC.1

PWM2_start

'START MAINLOOP

'------------

Lus:

ReadValues      'Spier1,2,3 and Stijfheid and frequentie are read and convert to the right values

Display         'puts the values on the Display

' SWITCH STATUS CHECK

'------------------------

if PORTC.0 =1 then

DemoKnop=1

ManKnop=0

NopKnop=0

Lcd8_out(4,-3,"                ")

Count = 0

A=0

V=0

PWM1_change_duty(0)

PWM2_change_duty(0)

end if

if PORTC.3=1 then

DemoKnop=0

ManKnop=1

NopKnop=0

Lcd8_out(4,-3,"                ")

Count = 0

A=0

V=0

PWM1_change_duty(0)

PWM2_change_duty(0)

end if

if PORTC.4=1 then

DemoKnop=0

ManKnop=0

NopKnop=1

end if

if  DemoKnop=1 then

goto Demo

end if

if  ManKnop=1 then

goto manual

end if

if  NopKnop=1 then

goto NOP_lus

end if

NOP_lus:

Lcd8_out(4,-3,"                ")

Count = 0

A=0

V=0

PWM1_change_duty(0)

PWM2_change_duty(0)

goto Lus

Demo:

Lcd8_out(4,7," Demo")

if  Fr<300 then

T=10

periode=60

Lcd8_out(4,-3,"Slow  ")

end if

if  Fr>=300 and (Fr<=400) then

T=25

periode=40

Lcd8_out(4,-3,"Medium")

end if

if  Fr>400 then

T=50

periode=25

Lcd8_out(4,-3,"Fast  ")

end if

Inc(Count)

If Count < periode then

if A<250 then

A=A+T

end if

PWM1_change_duty(0)

PWM2_change_duty(A)

B=1

end if

If Count > periode  then

if A=250 and(B=1) then

A=0

B=0

end if

if A<250 then

A=A+T

end if

PWM1_change_duty(A)

PWM2_change_duty(0)

end if

If Count > 2*periode then

Count = 0

A=0

end if

goto Lus

''methode 2

'   if A=0 and (V=0) then

'                A=255               ' spier 2 spier 3

'                V=0                 ' spier 1

'             end if

'

'

'                if (Hoek < Middle+20 ) and V=255 then

'                    A=0

'                    V=255

'                      PWM1_change_duty(A)

'                   PWM2_change_duty(V)

'                end if

'                if (Hoek  >= Middle+20) then

'                   A=0

'                   V=255

'                     PWM1_change_duty(A)

'                   PWM2_change_duty(V)

'                end if

'

'                if  (Hoek < Middle+20) and V=0 then

'                    A=255

'                    V=0

'                     PWM1_change_duty(A)

'                     PWM2_change_duty(V)

'                end if

'

'                if  (Hoek  <= Middle-20) then

'                      A=255

'                      V=0

'                        PWM1_change_duty(A)

'                         PWM2_change_duty(V)

'                end if

' goto Lus

Manual:

Lcd8_out(4,7,"Manual")

Lcd8_out(4,0,stifness)      'value of Hoek

Lcd8_out(4,-3,"Stif:")

Lcd8_out(4,5,"%")

if A>(Manual) then

A=A-10

end if

if A<(Manual) then

A=A+ 10

end if

PWM2_change_duty(A)   'voorste spier

PWM1_change_duty(A)

'       if A<250 then

'                      A=50

'               end if

'

'               PWM1_change_duty(A)  'achterste spier

'               if Hoek < (Middle-10)  then

'

'                       j1=j1+50

'               end if

'               if Hoek < (Middle-5)   then

'                      j1=j1+5

'               end if

'

'               if Hoek >(Middle+20) then

'                      j1=j1-10

'               end if

goto Lus

end.